AIR POWER COMPANY

All Types of Nitrogen Gas Plant ,Oxygen Gas Plant Dryer ,Ammonia Cracker Air Dryer,Compressor &Vacuum Pumps Electrical Panel ,Capacitor Panel ,Fire Control Panel Manufacturing, AMC Contractor , Seal& Service, Suppliers

GST NO:- 09BAZPK5978D2ZT

Plot No-4, Block-9 , Pocket-7,Sector-82,Noida,201304.

Call at: 09810241427. Office:+919953252100,	Email : airpowercompanynoida15@gmail.com

PROPOSAL

QTN/APC/N2/26-27 /50	DATE: 10/06/2026

To,

M/s Apple

Plot No. …………………………,

Kind Attn: –   Mr Anil Sharma

(Manager- Production)

Reg : Nitrogen Gas Generator – 05 NM3/hr. capacity -99.5% Offer.

We refer to the discussions our Mr. Anil Sharma had with you regarding your Requirement of an on-site Nitrogen gas generator for meeting your high purity Nitrogen gas in sealed quench furnaces. We take pleasure in submitting you our Offer as under:

We are an engineering organization specializing in the design, Manufacture and turnkey supply of

On- site Gas Systems.

Compressed Air Systems 'CAS'.

- Gas & Moisture Analyzers.

- Air / Gas Storage Tanks.

We have more than 25 years of experience in the subject & about 350 gas plant installations within and outside the country. We sincerely work towards increasing the overall reliability of 'On site gas plant' technology by constantly working towards Improving designs, operating efficiencies, maintenance friendly design and layouts and by introducing new features.

Nitrogen Gas Plant – Features

  - On site nitrogen generation from compressed air.

  - Standby  &  built  in  PSA  Nitrogen system.

  - Flow Meter for On Line.

  - Pre piped, cabled and a painted gas plant.

  - Maintenance friendly, compact gas plant.

  - Superior components for enhanced equipment performance over longer period of time.

  - Control Panel PLC Type shall have following feature.

Nitrogen Gas Plant Purity

As discussed, we are quoting to you for 99.5% pure nitrogen gas.

Gas Plant - Capacity

We are offering a system, which shall make available 05 NM3/hr. of Nitrogen. Flow of 05Nm3/hr and Maximum Pressure 5.5Kg/cm2 Surge Tank.

Fully Automatic Gas Plant

We are offering to you a fully automatic gas plant. The gas plant shall keep

Switching on / off based on Buffer Tank Pressure Low N2 Plant Start and Tank Pressure High N2 Plant Stop.

Our Offer

Kindly find our offer as per following Annexure:

Annexure I Technical offer. Annexure II Commercial offer Thanking you and looking forward for your response.

Yours faithfully,

Client: M/S. …………………………………..

Equipment: 'CMS 05' Nitrogen Generator.

Technical Specifications Alternate

Equipment: On site N2 Gas Plant Model: 'CMS 05'

Capacity: 05 NM3/hr.

Basis of Gas Generation: Continuous Plant Installation: Indoor

Product Gas Specification

Nitrogen: 99.5%

Oxygen: 00.1%

Oil: Nil

N2 plant generation pressure: 5.5 KG/cm2 Temperature: Ambient

PSA Nitrogen Module

PSA Nitrogen module consists of twin tower arrangement filled with activated Alumina & a special grade of Carbon Molecular Sieves (C.M.S.), which have Preferential adsorption for oxygen. While one tower remain under adsorption the Other undergoes regeneration / equalization automatically. For the purpose of quick Equalization, we have provided a reverse line equalization arrangement from

Nitrogen surge tank, which increase the operating efficiencies of the PSA nitrogen Module. At the nitrogen surge tank nitrogen gas of the above specifications shall be Made available, which is the product gas. Flow of product nitrogen gas is measured by installing a glass tube type rota-meter.

Oxygen Gas Analyzer- On Line Percentage

We have considered to supply an on line percentage oxygen analyzer in the product Nitrogen gas line. This analyzer shall have 2 alarm relays (one for alarm and the Other for activating Vent Valve), one of the relay is interlocked with the 3rd port of a Vent valve, provided in the product nitrogen gas line in a fashion that the 3rd port Shall open to atmosphere in case the Nitrogen purity from the plant drops below the Set limit. Line Diagram:

Scope of Supply

Our scope of supply shall be as under:

  - After Pre Filter-500microm with  Drain Valve	Make-Concept

  - Pre filters-                                                                   (Make-APC)

  - One stage heavy duty oil filter.                                  (Make-APC)

  - Fine Filter.                                                                    (Make-APC)

  - Twin tower based 'CMS05' Nitrogen module interconnected with set of Change over valves and piping.			 (Make-GE)

  - First fill of activated alumina Ceramic Boll.		 (Make-GE)

  - First fill of Carbon Molecular Sieves (C.M.S.).	 (Make-Germni)

  - Nitrogen surge tank with safety accessories.	 (Make-APC)

  - On line oxygen gas analyzer at surge tank.		 (Make-NUCON)

  - Pressure holding valve (P.H.V.) after nitrogen surge tank.

  - Rota meter (Metal Tube) with by- pass facility in raw nitrogen gas line.

  - Centralized electric control panel with following features:

  - Fault Indicator & PSA controller.

  - On line oxygen analyzer.

  - Painting of the equipment.

  - Commission of the equipment at site.

  - Nitrogen Gas storage tank at 5.5 kg/cm2 pressure complete with safety devices With orifice plate at outlet. (Optional).

Utilities Requirement

    - Power SUPPLY MCB-40A(4P)

415 Volt AC, 3 Phase, 50 Hz

    - Connected Lead 10 Kw

    - Consumed 0.4 kW per cubic meter Nitrogen generated

    - Screw Air Compressor 11kw with Dryer

    - Pipe Line for Compressor to N2 Plant

    - Earthing and In Comer Cabling as for Load

Space Requirement

We intend to install the proposed gas generator in linear fashion. Approx. space Requirement shall be as under:

Length (Left to Right)	: 2.0 Mtr. Width (Front to Back)	: 1.5 Mtr. Height (Bottom to Top)  : 1.8. Mtr. Installation	: Indoor

  - Approx. 550 mm maintenance spaces to be left on the left & right side and back Side of the installation.

  - Approx. 800 mm free space to be left in front of the gas plant installation.

  - Nitrogen Storage tank to be installed in the open.

  - Space required for Nitrogen Storage tank, not included in space indicated above.

Annexure – II (Commercial offer)

Model ‘’CMS 05’’ ,Capacity 05 NM3/hr & Purity-99.5%

  - ‘CMS' Nitrogen generator Along with an

    - PSA Tower 1 &2 with CMS - PSA Module.-01 Set.

    - Auto control Valving-08 Nos.

    - On line Oxygen Analyzer.-01 No.

    - Electrical Panel & Foult Indication-01Set.

    - Instrument Air Pot & Surge Tank-01 No.

    - Filter Unit with Element-01 No.

    - Safti Valve- 02 Nos.

    - Automatic Moisture Drain Valve-01 Nos.

    - Nitrogen Buffer Tank-01 Nos

    - Make- APC

…………………………………………………………………

Total Amount :-	Rs. 3,80,000.00 Amount in Words:- Three Lakh Eighty Thousand Only

Terms of Payment

    - 40% advance along with the Purchase order.

    - 60% advance along with taxes and duties against Proforma Invoice at our manufacturing

    - After successful Testing Insinuation & commissioning of the system.

    - If you will not used the system within 3 months , then you will have to pay 5% balance amount .

Delivery

Looking at your urgency, we shall offer delivery of the plant within 3 to 4 weeks from The date of receipt of formal order along with the advance payment.

Taxes & Duties

- Prices are ex works, Noida.

- Packing & Loading - @3% excluding

- GST-18%, excluding

- Octroi, Insurance extra at actual or client's scope.

- Freight will be excluding.

Erection & Commissioning

Cost included in the prices offered.

Guarantee

We shall extend following guarantees

  - Equipment against poor workmanship / faulty performance for a period of 12 Months from the date of commissioning.

  - CMS topping requirement for a period of 2 years.

  - CMS will be guaranteed for a period of 2 years though the expected life will be

  - Many more years.